Smart Irrigation System

ESP32 — IoT University Project

Youssef Gaied | ISITCOM Tunisia | April 2026

# 1. Project Overview

This project is a smart automatic irrigation system built on the ESP32 microcontroller. It monitors soil moisture and waters plants automatically, while providing remote monitoring and control through an Android application powered by Firebase. An AI agent (n8n + LLM) provides intelligent irrigation decisions.

## System Architecture

ESP32 (sensors + pump)  →  Firebase Realtime Database  →  Android App

Android App (manual mode)  →  Firebase  →  MQTT  →  ESP32

n8n Agent (auto mode)  →  LLM decision  →  MQTT  →  ESP32

## Key Features

- Soil moisture monitoring with automatic irrigation

- Temperature and humidity sensing (DHT22)

- Light level detection (LDR) to avoid watering at wrong times

- Firebase cloud integration for real-time data

- Android app with Manual and Auto irrigation modes

- AI agent (n8n + LLM) for intelligent irrigation decisions

- MQTT for reliable push commands to ESP32

# 2. Hardware Components

| Component | Details | Qty | Status |
| --- | --- | --- | --- |
| ESP32 DevKit v1 | USB-C, CP2102, 38 pins | 1 | ✅ |
| Soil Moisture Sensor v1.2 | Capacitive, black PCB, 3-wire | 1 | ✅ |
| DHT22 Module | Temp + Humidity, 3 pins | 1 | ✅ |
| 2-Channel Relay Module | 5V, opto-isolated, active LOW | 1 | ✅ |
| Mini Submersible Pump | 3-6V DC, tested working | 1 | ✅ |
| LDR 5mm | Photoresistance | 2 | ✅ |
| 10KΩ Resistors | 1/4W | 5 | ✅ |
| 830-point Breadboard | White, full size | 1 | ✅ |
| Jumper Wires | M-M long + M-F short | set | ✅ |
| USB-C Data Cable | ttyUSB0 confirmed | 1 | ✅ |

# 3. Wiring & GPIO Assignment

| GPIO | Board Label | Connected To | Notes |
| --- | --- | --- | --- |
| GPIO 34 | D34 | Soil sensor AOUT | Analog input |
| GPIO 4 | D4 | DHT22 DATA | Digital 1-wire |
| GPIO 26 | D26 | Relay IN1 | Digital output, active LOW |
| GPIO 35 | D35 | LDR junction | Analog input |
| GPIO 21 | D21 | OLED SDA (future) | I2C |
| GPIO 22 | D22 | OLED SCL (future) | I2C |
| 3V3 | 3V3 | Soil sensor VCC, DHT22 VCC | 3.3V power |
| VIN | VIN | Relay JD-VCC, VCC, NO terminal | 5V power |
| GND | GND | All grounds, pump BLACK | Common ground |

## Critical Wiring Rules

- Soil sensor VCC = 3.3V ONLY — 5V will damage ESP32 ADC permanently

- Relay VCC/JD-VCC = VIN (5V) ONLY — 3.3V will not activate the coil

- Pump BLACK wire goes to GND directly, NOT through the relay

- Relay is ACTIVE LOW: LOW = pump ON, HIGH = pump OFF

- Always set GPIO 26 HIGH in setup() to prevent pump running on boot

- Use 2.4GHz WiFi — ESP32 does not support 5GHz

## LDR Voltage Divider (on breadboard)

3V3 ──── LDR leg 1

LDR leg 2 ──── GPIO 35 (AND top of 10KΩ)

10KΩ resistor

GND

## Relay & Pump Wiring

- Relay JD-VCC → VIN (5V)

- Relay VCC → VIN (5V)

- Relay GND → GND

- Relay IN1 → GPIO 26

- Relay COM terminal → Pump RED wire

- Relay NO terminal → VIN (5V)

- Pump BLACK wire → GND directly

# 4. Software Setup

## Arduino IDE Configuration

- Arduino IDE 2.x installed on Linux (Ubuntu)

- Launch command: ./arduino-ide_*.AppImage --no-sandbox

- Board: ESP32 Dev Module

- Port: /dev/ttyUSB0

- CP2102 chip — no extra drivers needed on Linux

- Permission fix: sudo usermod -a -G dialout $USER

## Libraries Installed

- DHT sensor library v1.4.7 by Adafruit

- Adafruit Unified Sensor v1.1.15 by Adafruit

## Uploading Tips

- No need to press BOOT button — board auto-uploads

- Each new upload completely replaces the previous code

- Open Serial Monitor at 115200 baud to see sensor output

# 5. Progress Status

| Step | Task | Status |
| --- | --- | --- |
| Step 1 | Component list + procurement | ✅ DONE |
| Step 2 | Wiring diagram + connections | ✅ DONE |
| Step 3 | Arduino IDE setup | ✅ DONE |
| Step 4 | Blink test — LED confirmed blinking | ✅ DONE |
| Step 5 | Relay + pump test — pump cycles 3s ON / 3s OFF | ✅ DONE |
| Step 6 | All sensors reading on Serial Monitor | ✅ DONE |
| Step 7 | Soil sensor calibration | ✅ DONE |
| Step 8 | ESP32 → Firebase (send data every hour) | ⏳ NEXT |
| Step 9 | Android app (live data + manual pump button) | ⏳ PENDING |
| Step 10 | MQTT on ESP32 (listen for irrigate command) | ⏳ PENDING |
| Step 11 | n8n local setup + LLM agent | ⏳ PENDING |
| Step 12 | Full system end-to-end test | ⏳ PENDING |
| Step 13 | Portfolio documentation | ⏳ PENDING |

# 6. Soil Sensor Calibration Results

Calibration performed with capacitive soil moisture sensor v1.2 on GPIO 34.

SOIL_DRY = 2615   // sensor in open air (range: 2605 - 2625)

SOIL_WET =  926   // sensor tip in water (range: 925 - 928)

Moisture % = map(rawValue, 2615, 926, 0, 100)

constrain result to 0-100%

# 7. Code Reference

## Code 1 — Blink Test

Purpose: Confirms Arduino IDE can upload code to the ESP32. Blinks the onboard LED (GPIO 2) every 500ms.

void setup() {

pinMode(2, OUTPUT);  // GPIO 2 = onboard LED

}

void loop() {

digitalWrite(2, HIGH);  // LED ON

delay(500);

digitalWrite(2, LOW);   // LED OFF

delay(500);

}

## Code 2 — Relay & Pump Test

Purpose: Tests the relay module and pump. Cycles pump ON for 3 seconds, OFF for 3 seconds. Always put pump in water before running.

#define RELAY_PIN 26

void setup() {

Serial.begin(115200);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, HIGH);  // pump OFF at start

Serial.println("Relay test starting...");

}

void loop() {

Serial.println("Pump ON");

digitalWrite(RELAY_PIN, LOW);   // active LOW = pump ON

delay(3000);

Serial.println("Pump OFF");

digitalWrite(RELAY_PIN, HIGH);  // pump OFF

delay(3000);

}

## Code 3 — All Sensors Reading

Purpose: Reads all 4 sensors simultaneously and prints to Serial Monitor every 2 seconds. Used for initial sensor verification.

#include <DHT.h>

#define SOIL_PIN    34

#define DHT_PIN      4

#define RELAY_PIN   26

#define LDR_PIN     35

#define DHT_TYPE  DHT22

DHT dht(DHT_PIN, DHT_TYPE);

void setup() {

Serial.begin(115200);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, HIGH);  // pump OFF at boot

dht.begin();

Serial.println("Smart Irrigation System starting...");

}

void loop() {

int soilRaw = analogRead(SOIL_PIN);

float temp  = dht.readTemperature();

float humi  = dht.readHumidity();

int light   = analogRead(LDR_PIN);

Serial.print("Soil Raw: ");  Serial.println(soilRaw);

Serial.print("Temp: ");      Serial.print(temp);   Serial.println(" C");

Serial.print("Humidity: ");  Serial.print(humi);   Serial.println(" %");

Serial.print("Light: ");     Serial.println(light);

Serial.println("---");

delay(2000);

}

## Code 4 — Full Sensor Code with Auto Irrigation

Purpose: Main irrigation logic. Reads all sensors, converts soil raw value to moisture %, and automatically triggers the pump when moisture falls below 30%. Uses real calibration values.

#include <DHT.h>

#define SOIL_PIN    34

#define DHT_PIN      4

#define RELAY_PIN   26

#define LDR_PIN     35

#define DHT_TYPE  DHT22

#define SOIL_DRY  2615   // calibrated: sensor in air

#define SOIL_WET   926   // calibrated: sensor in water

#define MOISTURE_THRESHOLD 30  // irrigate if below 30%

DHT dht(DHT_PIN, DHT_TYPE);

void setup() {

Serial.begin(115200);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, HIGH);  // pump OFF at boot

dht.begin();

Serial.println("Smart Irrigation System starting...");

}

int getMoisturePercent(int raw) {

int pct = map(raw, SOIL_DRY, SOIL_WET, 0, 100);

return constrain(pct, 0, 100);

}

void loop() {

int soilRaw  = analogRead(SOIL_PIN);

int moisture = getMoisturePercent(soilRaw);

float temp   = dht.readTemperature();

float humi   = dht.readHumidity();

int light    = analogRead(LDR_PIN);

Serial.print("Moisture: "); Serial.print(moisture); Serial.println("%");

Serial.print("Temp: ");     Serial.print(temp);     Serial.println(" C");

Serial.print("Humidity: "); Serial.print(humi);     Serial.println(" %");

Serial.print("Light: ");    Serial.println(light);

Serial.println("---");

if (moisture < MOISTURE_THRESHOLD) {

Serial.println(">> Soil DRY -- pump ON");

digitalWrite(RELAY_PIN, LOW);

delay(3000);

digitalWrite(RELAY_PIN, HIGH);

Serial.println(">> Watering done");

}

delay(2000);

}

# 8. Next Steps

## Step 8 — Firebase Integration

- Create Firebase project at console.firebase.google.com

- Enable Realtime Database

- Install Firebase ESP32 library

- ESP32 connects to WiFi and sends sensor data every hour

- Data structure: /irrigation/moisture, /temp, /humidity, /light

## Step 9 — Android App

- Built in Android Studio

- Reads live data from Firebase Realtime Database

- Manual mode: user taps Irrigate button → writes command to Firebase → MQTT → ESP32

- Auto mode: n8n agent handles irrigation decisions

- Toggle switch to switch between Manual and Auto mode

## Step 10 — MQTT on ESP32

- ESP32 subscribes to MQTT topic: irrigation/command

- Listens for message: IRRIGATE or STOP

- Executes pump accordingly

## Step 11 — n8n Agent + LLM

- n8n installed locally on Ubuntu

- Scheduled workflow: reads Firebase data every hour

- Sends data to LLM with context about Tunisian climate

- LLM decides: irrigate yes/no based on moisture, temp, humidity, light

- n8n publishes MQTT command to ESP32

## Advanced Portfolio Ideas

- Weather API (OpenWeatherMap) — skip watering if rain expected

- OLED display (SSD1306, I2C on GPIO 21+22) — local status display

- Multiple soil sensors (GPIO 32, 33) — multi-zone irrigation

- Solar panel + LiPo battery — off-grid operation

- Data logging to SD card in CSV format

Smart Irrigation System — ISITCOM Tunisia | Youssef Gaied | April 2026